ReadMe Assignment - 1

UART Telemetry System with Sensor Simulation

Overview

This project implements a real-time UART telemetry pipeline that simulates sensor data, packages it into structured telemetry frames, and receives, validates, and logs the data on a ground-station side.

The objective of this assignment was not only to transmit data, but to design a robust, debuggable telemetry system similar to what is used in embedded aerospace and UAV applications.

Sensor Behaviour

- Accelerometer and gyroscope readings follow idealized sinusoidal patterns to simulate motion.

- Altitude increases linearly over time.

- Temperature fluctuates minimally around a nominal value.

Communication Protocol

- UART communication is assumed error-free except for the simulated checksum verification.

- Frame size and structure are fixed and do not adapt dynamically.

System Constraints

- The system is single-threaded; real-time constraints are relaxed for simulation purposes.

- No physical sensors are connected; all data is software-generated.

Files

1. Sensor Data Generation (sensor_sim.c)

This module simulates multiple onboard sensors:

- Accelerometer (ax, ay, az)

- Gyroscope (gx, gy, gz)

- Altitude

- Temperature

Mathematical functions are used to generate smooth, time-varying data to emulate real sensor behaviour.

2. Telemetry Framing

Each telemetry packet follows a structured format:

$<FrameType>,<Timestamp>,<Sensor Data>*<Checksum>

Features:

- Start marker for frame synchronization

- Comma-separated payload

- XOR checksum for data integrity

- Fixed transmission rate (~20 Hz)

This framing approach is inspired by commonly used protocols such as NMEA and UAV telemetry links.

3. UART Communication

Since no physical UART hardware was available during development, UART communication was emulated using standard input/output streams.

- The C telemetry program writes frames to stdout

- The Python receiver reads frames from stdin

- Shell piping is used to connect both programs

This approach allows full protocol validation and testing without hardware dependency.

Execution command:

./telemetry_tx |python receiver.py

4. Ground Station Receiver (rt.py)

The Python script acts as a ground station and performs:

- Continuous stream reading

- Frame boundary detection

- Checksum verification

- Payload parsing

- Console visualization

- CSV data logging

All valid telemetry frames are stored in telemetry_log.csv for offline analysis.

CSV Logging

Telemetry data is logged into a CSV file with labeled columns, making it compatible with:

- Excel / LibreOffice

- MATLAB

- Python (pandas)

- Data visualization tools

This simulates post-flight or post-test data analysis workflows used in real systems.

Issues Encountered and Solutions

1. Serial module errors

- Issue: serial.Serial attribute errors

- Cause: Missing or conflicting Python modules

- Solution: Corrected environment setup and module usage

2. COM port not found

- Issue: No physical UART device available

- Solution: Emulated UART using stdin/stdout piping

3. No output in receiver

- Issue: Output buffering on Windows

- Solution: Disabled buffering using setbuf(stdout, NULL)

4. Receiver waiting indefinitely

- Issue: Python receiver run without piped input

- Solution: Correct execution using shell piping

AI

AI-assisted features available in VS Code Insiders were used during development for:

- Code formatting suggestions

- Syntax checks

- Debugging assistance

Summary

This project demonstrates:

- Embedded-style telemetry design

- Real-time data streaming

- UART protocol handling

- Checksum-based validation

- Ground-station data processing

- Debugging of stream-based systems

The implementation prioritizes clarity, reliability, and explainability, aligning with real-world embedded development practices.

How to Run

gcc telemetry_tx.c -o telemetry_tx

./telemetry_tx| python receiver.py